# Design of two Airfoils for a Canard Airplane

Martin Hepperle

For the airplane-project 'YAKA' (a development of the Ecole Supériore at Toulouse, France) two airfoils have been developed: Section MH200 for application in the main wing and section MH201 for the canard wing.

All results were obtained by using the Eppler-Code [1] for design and analysis of the airfoils. The design method of this code is based on conformal mapping while the analysis of various flap deflections is performed by a higher order panel method. Once the velocity-distribution is known, the drag-coefficients are calculated using an integral boundary layer analysis method. The lift- and momentum-coefficients from the potential theory are corrected for boundary layer effects (especially separation).

## Section MH 200

The airfoil MH200 was designed to surpass the aerodynamic characteristics of the NACA 631-412 section while conserving maximum lift coefficient, momentum coefficient and at least retaining the thickness of 12 percent.

The results show some improvements:

- the increased thickness of approximatly 13 percent allows a weight reduction of the main wing structure.

- the laminar-bucket characteristics of the NACA 631-412 were avoided by incorporating a smooth change of transition location with variation of angle of attack .

- the drag polars show improvements in the Reynoldsnumber range between 1·106 and 8·106 which should lead to better cruise performance of the entire aircraft. These effects were achieved without reducing the maximum lift coefficient or increasing the momentum coefficient.

- positive flap deflections up to 10 degrees allow shifting the region of minimum drag to a lift coefficient cl = 1.0 while performing better than the NACA 631-412 airfoil.

- the same is true for negative flap deflections, moving the low drag region to cl = -0.3 at a flap deflection of  = -10°, thus resulting in lower drag in case of aileron and flap deflections.

## Section MH 201

The airfoil MH201 was intended to replace the NASA GA(W)-2 airfoil. The airfoil shows a smoother drag polar at the same or lower drag coefficients than the GA(W)-2. Flap deflections are more effective when compared to the NASA airfoil.

## Bibliography

[1]	R.Eppler, D.Somers: ‘A Computer Program for the Design and Analysis of Low-Speed Airfoils’, NASA TM-80210, 1980

[2]	R.J.Mc Ghee, W.D.Beasley: ‘Low Speed Aerodynamic Characteristics of a 13 Percent thick Airfoil Section designed for General Aviation Applications’, NASA TM X-72697, 1975

Fig.1: Three view drawing of the ‘YAKA’ project.

Fig.2: Velocity distributions and drag polars of NACA 631-412, smooth surface.

Fig.3: Velocity distributions and drag polars of MH 200, smooth surface.

Fig.4: Drag polars of NACA 631-412, rough surface.

Fig.5: Drag polars of MH 200, rough surface.

Fig.6: Drag polars of MH 200 with a flap deflection of +5°, smooth surface.

Fig.7: Drag polars of MH 200 with a flap deflection of +5°, rough surface.

Fig.8: Drag polars of MH 200 with a flap deflection of -5°, smooth surface.

Fig.9: Drag polars of MH 200 with a flap deflection of -5°, rough surface.

Fig.10: Drag polars of NASA GA(W)-2, smooth surface.

Fig.11: Drag polars of MH 201, smooth surface.

Fig.12: Drag polars of MH 201 with a flap deflection of +5°, smooth surface.

Fig.13: Drag polars of MH 201 with a flap deflection of -5°, smooth surface.